实验七 无线数据传输 实验报告

| 姓名 | 班级 | 学号 | 指导老师 |
| --- | --- | --- | --- |
| 上官鸿 | 计算1614 | 201621121110 | 刘晋明 |

- 实验目的

- 掌握SPI总显得使用方式

- 掌握NRF24L01的工作原理和使用方式

- 掌握无线传输数据的基本原理

- 实验内容

学习NRF24L01的工作原理，利用总线技术实现两台单片机之间的AD转换数据的无线传输

- 实验步骤

- 编写NRF24L01的读写程序

nrf24l01.h :

#ifndef __NRF24L01_H_

#define __NRF24L01_H_

#include "reg52.h"

// Define interface to nRF24L01

// Define SPI pins

sbit CE = P1 ^ 0;   // Chip Enable pin signal (output)

sbit CSN = P3 ^ 7;  // Slave Select pin, (output to CSN, nRF24L01)

sbit IRQ = P3 ^ 3;  // Interrupt signal, from nRF24L01 (input)

sbit MISO = P2 ^ 0; // Master In, Slave Out pin (input)

sbit MOSI = P2 ^ 2; // Serial Clock pin, (output)c

sbit SCK = P3 ^ 6;  // Master Out, Slave In pin (output)

#ifndef uchar

#define uchar unsigned char

#endif

#ifndef uint

#define uint unsigned int

#endif

#define TX_ADR_WIDTH 5    // 5 uints TX address width

#define RX_ADR_WIDTH 5    // 5 uints RX address width

#define TX_PLOAD_WIDTH 5 // 5 uints TX payload

#define RX_PLOAD_WIDTH 5 // 5 uints TX payload

// SPI(nRF24L01) commands

#define READ_REG 0x00    // Define read command to register

#define WRITE_REG 0x20   // Define write command to register

#define RD_RX_PLOAD 0x61 // Define RX payload register address

#define WR_TX_PLOAD 0xA0 // Define TX payload register address

#define FLUSH_TX 0xE1    // Define flush TX register command

#define FLUSH_RX 0xE2    // Define flush RX register command

#define REUSE_TX_PL 0xE3 // Define reuse TX payload register command

#define NOP 0xFF         // Define No Operation, might be used to read status register

// SPI(nRF24L01) registers(addresses)

#define CONFIG 0x00      // 'Config' register address

#define EN_AA 0x01       // 'Enable Auto Acknowledgment' register address

#define EN_RXADDR 0x02   // 'Enabled RX addresses' register address

#define SETUP_AW 0x03    // 'Setup address width' register address

#define SETUP_RETR 0x04  // 'Setup Auto. Retrans' register address

#define RF_CH 0x05       // 'RF channel' register address

#define RF_SETUP 0x06    // 'RF setup' register address

#define STATUS 0x07      // 'Status' register address

#define OBSERVE_TX 0x08  // 'Observe TX' register address

#define CD 0x09          // 'Carrier Detect' register address

#define RX_ADDR_P0 0x0A  // 'RX address pipe0' register address

#define RX_ADDR_P1 0x0B  // 'RX address pipe1' register address

#define RX_ADDR_P2 0x0C  // 'RX address pipe2' register address

#define RX_ADDR_P3 0x0D  // 'RX address pipe3' register address

#define RX_ADDR_P4 0x0E  // 'RX address pipe4' register address

#define RX_ADDR_P5 0x0F  // 'RX address pipe5' register address

#define TX_ADDR 0x10     // 'TX address' register address

#define RX_PW_P0 0x11    // 'RX payload width, pipe0' register address

#define RX_PW_P1 0x12    // 'RX payload width, pipe1' register address

#define RX_PW_P2 0x13    // 'RX payload width, pipe2' register address

#define RX_PW_P3 0x14    // 'RX payload width, pipe3' register address

#define RX_PW_P4 0x15    // 'RX payload width, pipe4' register address

#define RX_PW_P5 0x16    // 'RX payload width, pipe5' register address

#define FIFO_STATUS 0x17 // 'FIFO Status Register' register address

void Delay1ms(uint y);

uchar SPI_RW(uchar byte);

uchar SPI_RW_Reg(uchar reg, uchar value);

uchar SPI_Read(uchar reg);

uchar SPI_Read_Buf(uchar reg, uchar *pBuf, uchar bytes);

uchar SPI_Write_Buf(uchar reg, uchar *pBuf, uchar bytes);

void NRF24L01_Config(void);

uchar NRF24L01_RxPacket(uchar *rx_buf);

void NRF24L01_TxPacket(uchar * tx_buf);

void RX_Mode(void);

void TX_Mode(void);

#endif

nrf24l01.c :

#include "nrf24l01.h"

uchar TX_ADDRESS[TX_ADR_WIDTH] = {0xE7, 0xE7, 0xE7, 0xE7, 0xE7}; //本地地址

uchar RX_ADDRESS[RX_ADR_WIDTH] = {0xE7, 0xE7, 0xE7, 0xE7, 0xE7}; //接收地址

sbit led7 = P1^7;

/*******************************************************************************

* 函 数 名         : Delay1ms

* 函数功能           : 延时函数

* 输    入         : 无

* 输    出         : 无

*******************************************************************************/

void Delay1ms(uint y) {

uint x;

for (; y > 0; y--) {

for (x = 110; x > 0; x--);

}

}

/**************************************************

Function: SPI_RW();

Description:

Writes one byte to nRF24L01, and return the byte read

from nRF24L01 during write, according to SPI protocol

假设下面的8位寄存器装的是待发送的数据10101010，上升沿发送、下降沿接收、高位先发送。

那么第一个上升沿来的时候 数据将会是sdo=1；

寄存器中的10101010左移一位，后面补入送来的一位未知数x，成了0101010x。

下降沿到来的时候，sdi上的电平将锁存到寄存器中去，那么这时寄存器=0101010sdi，

这样在 8个时钟脉冲以后，两个寄存器的内容互相交换一次。这样就完成里一个spi时序

/*  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  */

uchar SPI_RW(uchar byte) {

uchar bit_ctr;

for (bit_ctr = 0; bit_ctr < 8; bit_ctr++) {                       // output 8-bit    // 输出 8 位

MOSI = (byte & 0x80); // output 'byte', MSB to MOSI        // 从MOSI脚输出,从高位到低位.当uchar 最高位为1时逻辑运算为1,输出1.运算为0时输出0

byte = (byte << 1); // shift next bit into MSB..        // 左移1位.将低位向高位转移.

SCK = 1;              // Set SCK high..    // 将SCK脚拉高,

byte |= MISO;         //byte=byte|MISO  capture current MISO bit        // 从MISO 中读出状态位存入uchar中.

SCK = 0;              // ..then set SCK low again       // 将SCK脚拉低.发送一位完成.

}

return (byte); // return read byte      // 返回状态位

}

/**************************************************/

/**************************************************

Function: SPI_RW_Reg();

Description:

Writes value 'value' to register 'reg'

/*  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  */

uchar SPI_RW_Reg(uchar reg, uchar value) {

uchar status;

CSN = 0;              // CSN low, init SPI transaction   // CS为低,开始SPI传输

status = SPI_RW(reg); // select register  // 传送命令与地址

SPI_RW(value);        // ..and write value to it..   // 写1 uchar 值

CSN = 1;              // CSN high again    // 完成SPI传输

return (status); // 返回 status        // return nRF24L01 status byte

}

/**************************************************/

/**************************************************

Function: SPI_Read();

Description:

Read one byte from nRF24L01 register, 'reg'

/*  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  */

uchar SPI_Read(uchar reg) {

uchar reg_val;

CSN = 0;             // CSN low, initialize SPI communication...  // CS为低,开始SPI传输

SPI_RW(reg);         // Select register to read from..  // 设置读地址

reg_val = SPI_RW(0); // ..then read registervalue     // 读数据

CSN = 1;             // CSN high, terminate SPI communication   // 完成SPI传输

return (reg_val); // return register value  // 返回数据

}

/**************************************************/

/**************************************************

Function: SPI_Read_Buf();

Description:

Reads 'bytes' of bytes from register 'reg'

Typically used to read RX payload, Rx/Tx address

/**************************************************/

uchar SPI_Read_Buf(uchar reg, uchar * pBuf, uchar bytes) {

uchar status, byte_ctr;

CSN = 0;              // CS为低,开始SPI传输// Set CSN low, init SPI tranaction

status = SPI_RW(reg); // 传输读接收数据的命令.       // Select register to write to and read status byte

for (byte_ctr = 0; byte_ctr < bytes; byte_ctr++)

pBuf[byte_ctr] = SPI_RW(0); // 读 1 uchar数据   // Perform SPI_RW to read byte from nRF24L01

CSN = 1; // 数据传送完成. // Set CSN high again

return (status); // return nRF24L01 status byte

}

/**************************************************/

/**************************************************

Function: SPI_Write_Buf();

Description:

Writes contents of buffer '*pBuf' to nRF24L01

Typically used to write TX payload, Rx/Tx address

/**************************************************/

uchar SPI_Write_Buf(uchar reg, uchar * pBuf, uchar bytes) {

uchar status, byte_ctr;

CSN = 0;                                         // CS为低,开始SPI传输  // Set CSN low, init SPI tranaction

status = SPI_RW(reg);                            // 传输写发送数据的命令.   // Select register to write to and read status byte

for (byte_ctr = 0; byte_ctr < bytes; byte_ctr++)// 传送发送数据 // then write all byte in buffer(*pBuf)

SPI_RW( * pBuf++);

CSN = 1;         // 数据传送完成.     // Set CSN high again

return (status); // return nRF24L01 status byte

}

//配置函数

void NRF24L01_Config(void) {

//initial io

CE = 0; //芯片使能

CSN = 1; //SPI禁能

SCK = 0; //SPI时钟线拉高

CE = 0;

SPI_RW_Reg(WRITE_REG + CONFIG, 0X0F);               //上电模式，使能CRC(2个字节)和RX_DR使能

SPI_RW_Reg(WRITE_REG + EN_AA, 0X3f);                //频道0自动，ack应答允许

SPI_RW_Reg(WRITE_REG + EN_RXADDR, 0X3f);            //允许pipe0

SPI_RW_Reg(WRITE_REG + RF_CH, 0);                   //   设置信道工作为2.4GHZ，收发必须一致

SPI_RW_Reg(WRITE_REG + RF_SETUP, 0x07);                 //设置发射速率为1MHZ，发射功率为最大值0dB

SPI_RW_Reg(WRITE_REG + CONFIG, 0x0e);               // IRQ收发完成中断响应，16位CRC，主发送

SPI_RW_Reg(WRITE_REG + RX_PW_P0, RX_PLOAD_WIDTH);           //设置接收数据长度，本次设置为5字节

SPI_Write_Buf(WRITE_REG + TX_ADDR, TX_ADDRESS, TX_ADR_WIDTH);   // 写本地地址

SPI_Write_Buf(WRITE_REG + RX_ADDR_P0, RX_ADDRESS, RX_ADR_WIDTH); // 写接收端地址0

SPI_RW_Reg(WRITE_REG + CONFIG, 0x0e);               // IRQ收发完成中断响应，16位CRC，主发送

CE = 1;                                                 /////////////

}

/////接收数据

uchar NRF24L01_RxPacket(uchar * rx_buf) {

uchar revale = 0, sta;

SPI_RW_Reg(WRITE_REG + CONFIG, 0X0F);

CE = 1;

Delay1ms(10);

sta = SPI_Read(STATUS);

if (sta & 0x40) {//RX_DR = 1

CE = 0;

SPI_Read_Buf(RD_RX_PLOAD, rx_buf, RX_PLOAD_WIDTH);

revale = 1;

}

SPI_RW_Reg(WRITE_REG + STATUS, sta);

return revale;

}

void NRF24L01_TxPacket(uchar * tx_buf) {

CE = 0;

SPI_Write_Buf(WR_TX_PLOAD, tx_buf, TX_PLOAD_WIDTH); // Writes data to TX payload

CE = 1;

Delay1ms(10);

}

//发送数据

/**************************************************/

/**************************************************

Function: RX_Mode();

Description:

This function initializes one nRF24L01 device to

RX Mode, set RX address, writes RX payload width,

select RF channel, datarate & LNA HCURR.

After init, CE is toggled high, which means that

this device is now ready to receive a datapacket.

/*  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  */

void RX_Mode(void) {

CE = 0;

SPI_Write_Buf(WRITE_REG + RX_ADDR_P0, RX_ADDRESS, RX_ADR_WIDTH); // Use the same address on the RX device as the TX device

SPI_RW_Reg(WRITE_REG + EN_AA, 0x01);     // Enable Auto.Ack:Pipe0

SPI_RW_Reg(WRITE_REG + EN_RXADDR, 0x01); // Enable Pipe0

SPI_RW_Reg(WRITE_REG + RF_CH, 40);       // Select RF channel 40

SPI_RW_Reg(WRITE_REG + RX_PW_P0, RX_PLOAD_WIDTH); // Select same RX payload width as TX Payload width

SPI_RW_Reg(WRITE_REG + RF_SETUP, 0x07); // TX_PWR:0dBm, Datarate:2Mbps, LNA:HCURR

SPI_RW_Reg(WRITE_REG + CONFIG, 0x0f);   // Set PWR_UP bit, enable CRC(2 bytes) & Prim:RX. RX_DR enabled..

CE = 1; // Set CE pin high to enable RX device

Delay1ms(10);

//CE = 0;

//  This device is now ready to receive one packet of 16 bytes payload from a TX device sending to address

//  '3443101001', with auto acknowledgment, retransmit count of 10, RF channel 40 and datarate = 2Mbps.

}

/**************************************************/

/**************************************************

Function: TX_Mode();

Description:

This function initializes one nRF24L01 device to

TX mode, set TX address, set RX address for auto.ack,

fill TX payload, select RF channel, datarate & TX pwr.

PWR_UP is set, CRC(2 bytes) is enabled, & PRIM:TX.

ToDo: One high pulse(>10us) on CE will now send this

packet and expext an acknowledgment from the RX device.

/*  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  *  */

void TX_Mode(void) {

CE = 0;

SPI_Write_Buf(WRITE_REG + TX_ADDR, TX_ADDRESS, TX_ADR_WIDTH);   // Writes TX_Address to nRF24L01

SPI_Write_Buf(WRITE_REG + RX_ADDR_P0, RX_ADDRESS, RX_ADR_WIDTH); // RX_Addr0 same as TX_Adr for Auto.Ack

SPI_RW_Reg(WRITE_REG + EN_AA, 0x01);      // Enable Auto.Ack:Pipe0

SPI_RW_Reg(WRITE_REG + EN_RXADDR, 0x01); // Enable Pipe0

SPI_RW_Reg(WRITE_REG + SETUP_RETR, 0x1a); // 500us + 86us, 10 retrans...

SPI_RW_Reg(WRITE_REG + RF_CH, 40);        // Select RF channel 40

SPI_RW_Reg(WRITE_REG + RF_SETUP, 0x07); // TX_PWR:0dBm, Datarate:2Mbps, LNA:HCURR

SPI_RW_Reg(WRITE_REG + CONFIG, 0x0e);    // Set PWR_UP bit, enable CRC(2 bytes) & Prim:TX. MAX_RT & TX_DS enabled..

CE = 1;

//Delay1ms(10);

// CE = 0;

}

- 编写主控制程序，实现双向通信。

main.c :

#include "reg52.h" //此文件中定义了单片机的一些特殊功能寄存器

#include "nrf24l01.h"

#include "lcd.h"

#include "XPT2046.h"

typedef unsigned char u8;

typedef unsigned int u16;

sbit beep = P1^5;

uchar DATA[5] = {0}, rx_buf[5] = {0};

u16 cnt = 0, temp = 0;

int readData()

{

u16 t, change = 0;

if(cnt == 25)

{

cnt = 0;

t = Read_AD_Data(0x94);     // 电位器

if(t != temp) {

change = 1;

temp = t;

DATA[0]= temp/1000;//千位

DATA[1]= temp%1000/100;//百位

DATA[2]= temp%1000%100/10;//十位

DATA[3]= temp%1000%100%10;//个位

}

}

cnt ++;

return change;

}

uchar NRF24L01_Check(void)

{

uchar bu[5]={0XA5,0XA5,0XA5,0XA5,0XA5};

uchar bu1[5];

uchar i;

SPI_Write_Buf(WRITE_REG+TX_ADDR,bu,5);    //写入5个字节的地址.

SPI_Read_Buf(TX_ADDR,bu1,5);              //读出写入的地址

for(i=0;i<5;i++)if(bu1[i]!=0XA5)break;

if(i!=5)return 1;                               //NRF24L01不在位

return 0;                                       //NRF24L01在位

}

/*******************************************************************************

* 函 数 名       : main

* 函数功能         : 主函数

* 输    入       : 无

* 输    出         : 无

*******************************************************************************/

void main()

{

LcdInit();

NRF24L01_Config();

while(NRF24L01_Check())                 //检查不到24l01则报警

{

beep=0;

Delay1ms(20);

beep=1;

Delay1ms(20);

}

while (1)

{

RX_Mode();

if(NRF24L01_RxPacket(rx_buf)) {

LcdWriteCom(0x80);  //设置数据指针起点

LcdWriteData(rx_buf[0] % 10 + '0');

LcdWriteData(rx_buf[1] % 10 + '0');

LcdWriteData(rx_buf[2] % 10 + '0');

LcdWriteData(rx_buf[3] % 10 + '0');

}

if(readData()) {

TX_Mode();

NRF24L01_TxPacket(DATA);

}

}

}

- 调试

- 实验总结

- 掌握了SPI总线的使用方式

- 掌握了NRF24L01的工作原理和使用方式

- 掌握了无线传输的基本原理